Project Report for

RE4012 - MACHINE VISION

Irish Speed-Signs Detection and Classification

Revision Timestamp: 04/04/2025 03:58:18

Project Team

| Student Name | Student Id |
| --- | --- |
| ZEXIN LI | 23202637 |
| QINYUAN LIU | 20137095 |

Contents

Project Team	2

Contents	3

1.	Introduction	4

2.	Approach	4

3.	Mathematical principles	4

4.	Workflow of the Code	5

5.	Code Implementation:	7

6.	Results	13

7.	Conclusion	13

8.	References	15

- Introduction

Sinogram reconstruction is a common technique used in CT scanners, where one or multiple rotational beams of X-rays pass through an object, generating an image output on a sensing semiconductor. This output, commonly known as a sinogram, represents the raw projection data collected from different angles.

The sinogram produced by a CT scanner can be transformed into 2D cross-sectional slices of the scanned object, revealing its internal structure. This transformation is typically performed using Filtered Back Projection (FBP), a widely used reconstruction algorithm in medical imaging.

This project report focuses on the Python implementation of image reconstruction from a given sinogram (CT raw sinogram output) using the FBP algorithm. We will first explain the organizational approach taken in structuring the implementation, followed by a detailed discussion of the technical aspects of the project.

- Approach

The project is undertaking by 2 individuals, with attempt of recruit new members made. Unfortunately, we have not yet received a reply from suitable candidates. The project is implemented in 2 phases: Code development and Report writing. Zexin Li has undertaken the main part of the code development, and Qinyuan Liu has taken responsibility for code review and Report writing.

- Mathematical principles

Fourier transformation and inverse Fourier Transform are fundamental in Filtered Back Projection (FBP). The Fourier Transform helps in converting the sinogram projections into the frequency domain, allowing us to apply Ramp Filtering, which enhances high-frequency components and reduces blurring effects in the reconstructed image.

Ramp filtering is an essential step in FBP because direct back projection without filtering results in blurry reconstructions. The Ramp Filter acts as a high-pass filter that compensates for the excessive low-frequency components inherent in the projection data.

The following table shows the Mathematical Formula used and it’s associated code:

| Step | Mathematical Formula | Python Code |
| --- | --- | --- |
| 1D Fourier Transform | F(k)=∑f(n)e-j2πkn/N | F = rfft(row_data) |
| Ramp Filtering | Ffiltered​(k)=F(k)⋅k | k |
| Inverse Fourier Transform | f(n)=1N​∑Ffiltere​(k)ej2πkn/N | recon_image=iradon(filtered_sino, theta=theta, filter_name=None, circle=True) |
| Backprojection | I(x,y)=∫Sfiltered​(θ,p)dθ | recon_image = iradon(filtered_sino, theta=theta, filter_name=None, circle=True) |

Table 1 Formula Table

- Workflow of the Code

The workflow of the reconstruction process is as follows:

- Load sinogram

Table 2 Sinogram.png

Table 2 Sinogram.png

Read the sinogram file (sinogram.png) in RGB format, which usually consists of image projection data and is used for tomography reconstruction.

Figure 1 Code Workflow

Figure 1 Code Workflow

- Separate color channels

  - Split the RGB image into three independent channels: red (R), green (G), and blue (B), and process the projection data of each channel separately.

- Frequency domain stimulation

  - Apply a ramp filter (implemented in the frequency domain) to each channel. This step requires a Fourier transform (FFT) first, multiplying with the ramp filter (high-frequency compensation), and then converting it back to the spatial domain through an inverse Fourier transform (iFFT) to eliminate projection blur.

- Backprojection reconstruction

  - Project the auxiliary projection data into the image space and generate a two-dimensional reconstructed image for each channel by integration.

- Post-processing

- Crop region cancellation: remove the black border at the edge of the projected image (cancel region data).

- Adjust aspect ratio: correct the image ratio to avoid display distortion.

- Normalize to 8 bits: linearly scale the pixel values ​​to the range of 0-255, equipped with a standard image format.

- Merge color channels

  - Restore the brightness of the processed R, G, and B channels to an RGB color image.

- Save the final reconstructed image (final_recon_RGB.png) and display it.

- Code Implementation:

import numpy as np

import imageio.v3 as iio

import matplotlib.pyplot as plt

from numpy.fft import rfft, irfft

from skimage.transform import rotate, iradon

from skimage.transform import resize

def ramp_filter_1d_fftrow(row_data):

"""

1) rFFT

2) ramp

3) iFFT

"""

N = len(row_data)

#rFFT

F = rfft(row_data)  # shape: (N//2 + 1,)

#ramp

freq = np.arange(len(F))

F_filtered = F * freq

#irFFT

filtered_row = irfft(F_filtered, n=N)

return filtered_row

def hamming_window_1d(ffts):

hamming = np.hamming(ffts.shape[0])

return ffts * hamming

def ramp_filter_1d_hamming_fftrow(row_data):

# row_data must be 1d

N = len(row_data)

F = np.fft.rfft(row_data)

print(row_data.shape)

freq = np.fft.rfftfreq(N, d=1.0)

ramp = np.abs(freq)

F_filtered = F * ramp

F_filtered = hamming_window_1d(F_filtered)

filtered_row = irfft(F_filtered, n=N)

return filtered_row

def ramp_filter_1d_hann_fftrow(row_data):

N = len(row_data)

F = rfft(row_data)

freq = np.fft.rfftfreq(N, d=1.0)

ramp = np.abs(freq)

hann_win = np.hanning(len(F))

F_filtered = F * ramp * hann_win

filtered_row = irfft(F_filtered, n=N)

return filtered_row

def apply_filter_to_sinogram(sinogram_2d, filter_type='none'):

"""

use none, ramp, hamming, hann to specify which filter to use

"""

rows, cols = sinogram_2d.shape

output = np.zeros_like(sinogram_2d, dtype=np.float64)

if filter_type == 'none':

return sinogram_2d.astype(np.float64)

for i in range(rows):

row_data = sinogram_2d[i, :]

if filter_type == 'ramp':

output[i, :] = ramp_filter_1d_fftrow(row_data)

elif filter_type == 'hamming':

output[i, :] = ramp_filter_1d_hamming_fftrow(row_data)

elif filter_type == 'hann':

output[i, :] = ramp_filter_1d_hann_fftrow(row_data)

else:

output[i, :] = row_data

return output

def reconstruct_from_sinogram(sinogram_2d):

rows, cols = sinogram_2d.shape

theta = np.linspace(0., 180., cols, endpoint=False)

recon_image = iradon(sinogram_2d, theta=theta, filter_name=None, circle=False)

return recon_image

def crop_circle_region(image, threshold=0.01):

rows, cols = image.shape

mask = (image > threshold)

row_indices = np.where(np.any(mask, axis=1))[0]

col_indices = np.where(np.any(mask, axis=0))[0]

if len(row_indices) == 0 or len(col_indices) == 0:

return image

rmin, rmax = row_indices[0], row_indices[-1]

cmin, cmax = col_indices[0], col_indices[-1]

cropped = image[rmin:rmax + 1, cmin:cmax + 1]

return cropped

def apply_aspect_ratio(image, aspect_ratio_str):

if ':' in aspect_ratio_str:

w_ratio, h_ratio = aspect_ratio_str.split(':')

w_ratio = float(w_ratio)

h_ratio = float(h_ratio)

ratio = w_ratio / h_ratio

else:

ratio = 1.0

height, width = image.shape

new_width = int(round(height * ratio))

image_resized = resize(image, (height, new_width),

preserve_range=True,

anti_aliasing=False)

return image_resized

def float_to_8bit(image):

im_min, im_max = image.min(), image.max()

if im_max == im_min:

return np.zeros_like(image, dtype=np.uint8)

norm = (image - im_min) / (im_max - im_min)

out = (norm * 255.0).astype(np.uint8)

return out

metadata = iio.immeta("sinogram.png")

aspect_ratio_str = metadata.get("AspectRatio", "1:1")

sinogram_rgb = iio.imread('sinogram.png')

R, G, B = sinogram_rgb[:, :, 0], sinogram_rgb[:, :, 1], sinogram_rgb[:, :, 2]

iio.imwrite('sinogram_R.png', R)

iio.imwrite('sinogram_G.png', G)

iio.imwrite('sinogram_B.png', B)

sinogram_R = iio.imread("sinogram_R.png").T  # shape: (W, H)

sinogram_G = iio.imread("sinogram_G.png").T

sinogram_B = iio.imread("sinogram_B.png").T

# ========== (a) Reconstruction without Filtering ==========

sino_R_nofilter = apply_filter_to_sinogram(sinogram_R, filter_type='none')

sino_G_nofilter = apply_filter_to_sinogram(sinogram_G, filter_type='none')

sino_B_nofilter = apply_filter_to_sinogram(sinogram_B, filter_type='none')

recon_R_nofilter = reconstruct_from_sinogram(sino_R_nofilter)

recon_G_nofilter = reconstruct_from_sinogram(sino_G_nofilter)

recon_B_nofilter = reconstruct_from_sinogram(sino_B_nofilter)

# ========== (b) Reconstruction with Pure Ramp Filtering ==========

sino_R_ramp = apply_filter_to_sinogram(sinogram_R, filter_type='ramp')

sino_G_ramp = apply_filter_to_sinogram(sinogram_G, filter_type='ramp')

sino_B_ramp = apply_filter_to_sinogram(sinogram_B, filter_type='ramp')

recon_R_ramp = reconstruct_from_sinogram(sino_R_ramp)

recon_G_ramp = reconstruct_from_sinogram(sino_G_ramp)

recon_B_ramp = reconstruct_from_sinogram(sino_B_ramp)

# ========== (c) Reconstruction with Ramp Filtering + Hamming Window ==========

sino_R_hamming = apply_filter_to_sinogram(sinogram_R, filter_type='hamming')

sino_G_hamming = apply_filter_to_sinogram(sinogram_G, filter_type='hamming')

sino_B_hamming = apply_filter_to_sinogram(sinogram_B, filter_type='hamming')

recon_R_hamming = reconstruct_from_sinogram(sino_R_hamming)

recon_G_hamming = reconstruct_from_sinogram(sino_G_hamming)

recon_B_hamming = reconstruct_from_sinogram(sino_B_hamming)

# ========== (Optional) Reconstruction with Ramp Filtering + Hann Window ==========

sino_R_hann = apply_filter_to_sinogram(sinogram_R, filter_type='hann')

sino_G_hann = apply_filter_to_sinogram(sinogram_G, filter_type='hann')

sino_B_hann = apply_filter_to_sinogram(sinogram_B, filter_type='hann')

recon_R_hann = reconstruct_from_sinogram(sino_R_hann)

recon_G_hann = reconstruct_from_sinogram(sino_G_hann)

recon_B_hann = reconstruct_from_sinogram(sino_B_hann)

# ========== Post-processing: Crop, Adjust Aspect Ratio, Convert to 8-bit, and Merge into RGB ==========

def postprocess_and_merge(r_img, g_img, b_img, aspect_ratio_str):

R_crop = crop_circle_region(r_img)

G_crop = crop_circle_region(g_img)

B_crop = crop_circle_region(b_img)

R_resized = apply_aspect_ratio(R_crop, aspect_ratio_str)

G_resized = apply_aspect_ratio(G_crop, aspect_ratio_str)

B_resized = apply_aspect_ratio(B_crop, aspect_ratio_str)

R_8bit = float_to_8bit(R_resized)

G_8bit = float_to_8bit(G_resized)

B_8bit = float_to_8bit(B_resized)

min_rows = min(R_8bit.shape[0], G_8bit.shape[0], B_8bit.shape[0])

min_cols = min(R_8bit.shape[1], G_8bit.shape[1], B_8bit.shape[1])

R_final = R_8bit[:min_rows, :min_cols]

G_final = G_8bit[:min_rows, :min_cols]

B_final = B_8bit[:min_rows, :min_cols]

final_RGB = np.dstack([R_final, G_final, B_final])

return final_RGB

# Generate four sets of results (No Filtering / Ramp / Hamming / Hann)

final_nofilter_RGB = postprocess_and_merge(recon_R_nofilter, recon_G_nofilter, recon_B_nofilter, aspect_ratio_str)

final_ramp_RGB = postprocess_and_merge(recon_R_ramp, recon_G_ramp, recon_B_ramp, aspect_ratio_str)

final_hamming_RGB = postprocess_and_merge(recon_R_hamming, recon_G_hamming, recon_B_hamming, aspect_ratio_str)

final_hann_RGB = postprocess_and_merge(recon_R_hann, recon_G_hann, recon_B_hann, aspect_ratio_str)

# Save results

iio.imwrite("final_recon_no_filter.png", final_nofilter_RGB)

iio.imwrite("final_recon_ramp.png", final_ramp_RGB)

iio.imwrite("final_recon_hamming.png", final_hamming_RGB)

iio.imwrite("final_recon_hann.png", final_hann_RGB)

# ========== Display ==========

plt.figure(figsize=(12, 8))

plt.subplot(2, 2, 1)

plt.imshow(final_nofilter_RGB)

plt.title("No Filter Reconstruction")

plt.axis("off")

plt.subplot(2, 2, 2)

plt.imshow(final_ramp_RGB)

plt.title("Ramp Filter Reconstruction")

plt.axis("off")

plt.subplot(2, 2, 3)

plt.imshow(final_hamming_RGB)

plt.title("Ramp + Hamming Reconstruction")

plt.axis("off")

plt.subplot(2, 2, 4)

plt.imshow(final_hann_RGB)

plt.title("Ramp + Hann Reconstruction")

plt.axis("off")

plt.tight_layout()

plt.show()

print("Reconstruction results saved. Comparison displayed.")

- Results

Table 3 Reconstructed image

- Conclusion

The project demonstrates the students understanding of the FBP algorism, and the practical skills of the implementation in python environment.

The no-filter reconstruction illustrate the reconstructed image of sinogram.png, while the different windows (Filters) demonstrate that window function’s capability in digital processing.

Visually, the ramp filter provides the best outcome in terms of brain region distinction, while the Ramp + Hann is best for edge detection.

- References

- C. Flanagan, "Image Reconstruction from a Sinogram," RE4012/7 Machine Vision Project 1, 2024-5.

- C. Flanagan, "Radon Transform: Image Reconstruction from Projections," 2024-5.

- "Filtered Backprojection (FBP): Illustrated Guide for Radiologic Technologists," How Radiology Works, 2024. [Online]. Available: https://howradiologyworks.com/filtered-backprojection-fbp-illustrated-guide-for-radiologic-technologists/. [Accessed: 14-Mar-2025].

- [4] 谢钧, "现代医学成像（3）——CT（基本原理与图像重建）," 知乎, Jan. 24, 2020. [Online]. Available: https://zhuanlan.zhihu.com/p/90571757. [Accessed: Mar. 14, 2025].